Sex genotyping of mice

Cohn Group, GUDMAP Consortium

The sex genotyping method used by the Cohn lab is based on the highly homologous SMC genes found on the X (X chromosomal SmcX = Kdm5c) and Y (Y chromosomal SmcY = Kdm5d) chromosomes.

Homologous pair SmcX/SmcY

- SMCX-1 5'-CCGCTGCCAAATTCTTTGG-3'

- SMC4-1 5'-TGAAGCTTTTGGCTTTGAG-3'

- PCR product: females a single band (350 bp), males 2 bands (350 & 300 bp) because of an intron difference between the X and Y genes (Agulnik 1997, PMID 9060413) adapted from Case Western Transgenic Facility.

DNA extraction

1.  Add tissue, forelimb of embryo, to 300µl of 25mM NaOH.

2.  Heat at 95ºC for 2 hours.

3.  Add 300µl 40mM Tris HCl.

4.  Store at 4ºC till genotyping.

PCR  mix

1.  Set up the PCR mix as follows:

16.25µl    H20

5µl	     5X Green GoTaq Reaction Buffer (Promega cat #M791B)

1µl	     Primer X (CCGCTGCCAAATTCTTTGG, Integrated DNA            Technologies)

1µl	     Primer Y (TGAAGCTTTTGGCTTTGAG, Integrated DNA Technologies)

0.5µl	     dNTP mix ~ 10mM (Thermo Scientific cat #R0192)

0.25µl	     Taq DNA Polymerase with ThermoPol Buffer (New England BioLabs cat #M0267L)

1µl	     DNA

PCR Cycling Conditions

1.  94ºC  3:00 minutes   1 cycle

2.  94ºC     30 seconds   35 cycles

54ºC     30 seconds

72ºC     30 seconds

3.  72ºC  7:00 minutes	1 cycle

4.  10ºC  hold

Analysis of the PCR products on an agarose gel

1.  5µl of PCR product is run on a 2% agarose gel.

2.  Female mice are identified by a single 350bp band and males are identified by 2 smaller bands of 300bp & 350bp.