# In-situ Hybridization on Vibrating Microtome-cut Mouse Tissue Sections

Prior to starting

Prepare stock solutions, make DIG-labeled riboprobes, section mouse tissues with a vibrating microtome, make sample baskets and embryo powder as described below.

Make sample baskets

- 1. Use heated razor blade to cut off bottom of microcentrifuge tube (at or just below 100mL mark).

- 2. Cut polyester mesh into squares that are large enough to cover the cut part of the microfuge tube.

- 3. Heat cut edge of tube in flame until softened.

- 4. Press melted surface of microfuge tube firmly onto center of mesh square & hold for a few seconds to seal the mesh along entire circumference of tube.

- 5. Trim away excess mesh with scissors (may also heat near flame to melt down excess).

Make embryo powder

- 1. Collect mouse embryo tissue & store at –80°C until ready to make powder.

- 2. Place small amounts of frozen tissue into mortar, add liquid nitrogen & use pestle to grind tissue into powder (add more liquid nitrogen as needed).

- 3. Combine embryo powder with 4 volumes of acetone and homogenize with several strokes of dounce homogenizer until tissue is ground to a fine powder.

- 4. Transfer homogenate to a 15mL glass screw-top vial. Mix well and extract overnight at 4°C.

- 5. Pellet the embryo powder by centrifugation at 5000rpm for 10 min at 4°C.

- 6. Remove and discard supernatant.

- 7. Resupspend pellet in 4 volumes of fresh acetone and shake 2 hr at 4°C.

- 8. Pellet the embryo powder by centrifugation at 5000rpm for 10 min at 4°C.

- 9. Remove and discard supernatant.

- 10. Spread pellet onto #2 Whatman filter paper and air dry in fume hood.

- 11. Use mortar and pestle to grind dried pellet into fine powder.

- 12. Store powder in tightly sealed glass vial at 4°C.

DAY 1 – All incubation steps to be performed with gentile agitation on an orbital shaker, unless otherwise indicated.

- 1. Prep baskets

  - A. Close basket cap, affix sticker to cap and label with probe name.

  - B. Use heated 18 gauge needle to puncture two holes per basket cap (facilitates air movement).

- 2. Prepare supplies and preheat solutions.

  - A. Transfer prehyb stock solution into sealed conical tube and preheat to 60.5°C.

  - B. Fill a small plastic storage container (used as a hybridization chamber) with about ½ inch of tap water, cover container and preheat to 60.5°C.

- 3. Prep samples.

  - A. Use spring scissors to remove most of the agarose from the vibrating microtome-cut tissue sections.

  - B. Remove any debris if stuck to tissue sections.

  - C. Add 2mL PBSTw and one basket to each well of a 24-well culture plate.

  - D. Transfer sections into labeled baskets.

- 4. Incubate tissues for 30 min at 25°C in 2mL/well of 6% H2O2.

- 5. Wash tissues 4 x 5 min at 25°C with 2mL/well PBSTw.

- 6. Incubate tissues 12 min at 25°C in 2mL/well proteinase K solution.

- 7. Wash tissues 1 x 5 min at 25°C with 2mL/well PBSTw.

- 8. Incubate tissues 20 min at 25°C in 2mL/well post-fix solution.

- 9. Wash tissues 2 x 5 min at 25°C with 2mL/well PBSTw.

- 10. Prehybridization step: Incubate tissues inside hybridization chamber for at least 1 hr at 60.5°C in 2mL/well prehyb buffer.

- 11. Hybridization step: Add 0.65mg probe/well and incubate sections overnight in hybridization chamber at 60.5°C in probe+prehyb buffer.

- 12. Prepare and preheat solutions for Day 2: Add SDS to solution 1 (to a final concentration of 1%) and preheat overnight at 60.5°C.

DAY 2 - All incubation steps to be performed with gentile agitation on an orbital shaker, unless otherwise indicated.

- 1. Tissue sections are to remain in the baskets. Remove hyb buffer from each well and wash tissues 3 X 30 min at 60.5°C with 2mL/well Solution 1.

- 2. Prepare and preheat solutions.

  - A. Prepare 50/50% mix of Solution1/Solution 2 and preheat to 60.5°C.

  - B. Add RNase to Solution 2 and preheat to 37°C.

  - C. Preheat one wash volume of Solution 3 to 25°C and two wash volumes to 60.5°C.

- 3. Wash tissues 1 X 10 min at 60.5°C with 2mL/well of 50/50% mixture of Solution 1/ Solution 2.

- 4. Wash tissues 4 X 10 min at 25°C with 2mL/well Solution 2.

- 5. Incubate tissues 15 min at 37°C in 2mL/well RNase solution.

- 6. Wash tissues 1 X 10 min at 25°C with 2mL/well Solution 2.

- 7. Wash tissues 1 X 10 min at 25°C with 2mL/well Solution 3.

- 8. Wash tissues 2 X 1 hr at 60.5°C with 2mL/well Solution 3.

- 9. Preheat Tissue Blocking (TB), Antibody Dilution (AD), and Antibody Absorption buffers to 25°C.

- 10. Wash tissues 3 X 10 min at 25°C with 2ml/well TBSTw.

- 11. Blocking step

  - A. Incubate tissues at least 2 hr at 25°C in 2mL/well TB buffer.

  - B. Add 3.3mL anti-DIG antibody per 600µL AA buffer & incubate at least 2 hr at 4°C.

- 12. Antibody step

  - A. Centrifuge AA buffer containing antibody at 10,000 rpm for 1 min.

  - B. Remove supernatant and add entire supernatant volume to 6mL AD buffer.

  - C. In a humidified chamber, incubate tissues overnight at 4°C in 2mL AD buffer + antibody.

Day 3 - All incubation steps to be performed with gentile agitation on an orbital shaker, unless otherwise indicated.

- 1. Remove antibody solution and save.

  - A. Add sodium azide to a final concentration of 0.2mM to prevent microbial growth.

  - B. Store antibody solution at 4°C and reuse up to two additional times.

- 2. Wash tissues 8 X 10 min at 25°C with 2mL/well TBSTw containing 2 mM levamisole.

- 3. Remove tissues from baskets, use forceps to separate sections & remove visible debris, transfer tissues into clean microcentrifuge tubes.

- 4. Wash tissues 1 X 10 min at 25°C with 1mL NTMT containing 2 mM levamisole.

- 5. Detection step

  - A. Protect tissues from light and incubate at 25°C in 1mL/tube of a 60/40% mixture of NTMT containing 2mM levamisole/BM Purple.

  - B. Color development time ranges from several hours to several days. Change NTMT/BM Purple solution as needed (substrate will precipitate over time).

  - Once color is fully developed, wash tissues 2 X 5 min at 25°C with 1mL/tube NTMT containing 2mM levamisole.

- 6. Bleaching step

  - A. Post-fix tissues overnight at 4°C in 1mL/tube 4% PFA.

  - B. Remove PFA and incubate tissues for 30 min at 25°C in 1mL/tube PBSTw containing 3% H2O2.

  - C. Wash tissues 1 X 10 min at 25°C in 1mL/tube PBSTw.

  - D. Store tissues at 4°C in 1mL/tube 4% PFA.

## Solutions

Day 1

PBSTw: 1X PBS containing 0.1% Tween-20 and 0.2mM sodium azide. Pass through 0.2µm filter to remove insoluble material/contaminants.

6% H2O2: 1mL 30% H2O2 per 4mL PBS

Proteinase K: 0.25µL 20mg/mL proteinase K per 1mL PBSTw

Post-fix: 8µL 25% glutaraldehyde per 1mL 4% PFA

Prehybridization solution

| stock solution | final conc | for 1000mL |
| --- | --- | --- |
| 100% formamide | 50% | 500mL |
| 20X SSC | 5X | 250mL |
| Blocking reagent | 1% | 10g |
| 10mg/mL yeast tRNA | 10µg/mL | 1mL |
| 10mg/mL heparin | 10µg/mL | 1mL |
| dH2O to vol. | -- | to 1000mL |

Aliquot 50mL volumes into conical tubes & store at –20°C.

Solution 1

| stock solution | final conc | for 500mL |
| --- | --- | --- |
| 100% formamide | 50% | 250mL |
| 20X SSC | 5X | 125mL |
| dH2O | -- | 75mL |
| 10% SDS | 1% | 50mL |

Mix formamide, SSC & dH2O as shown above & store at –20°C.

DO NOT add SDS to freezer stock solution because SDS will precipitate in the cold.

Prior to using Solution 1 for ISH, add 1mL of 10% SDS per 9mL Solution 1 stock.

Day 2

Solution 2

| stock solution | final conc | for 500mL |
| --- | --- | --- |
| 1M Tris-HCl, pH 7.5 | 10mM | 5mL |
| 5M NaCl | 0.5M | 50mL |
| 100% Tween-20 | 0.1% | 0.5mL |
| 0.2M sodium azide | 0.2mM | 0.5mL |
| dH2O | -- | 444mL |

Pass through 0.2µm filter to remove insolubles/contaminants, store at 25°C.

RNase:  5µL RNase (50µg/mL) per 1mL Solution 2

Solution 3

| stock solution | final conc | for 500mL |
| --- | --- | --- |
| 20X SSC | 2X | 50mL |
| 100% formamide | 50% | 250mL |
| dH2O | -- | 200mL |

Store at –20°C

Tissue blocking (TB) buffer

| stock solution | final conc | for 500mL |
| --- | --- | --- |
| 10X TBS | 1X | 50mL |
| 100% sheep serum | 10% | 50mL |
| 10% blocking reagent | 1% | 50mL |
| BSA | 1% | 0.5g |
| dH2O to vol. | -- | to 500mL |
| 100% Tween-20 | 0.1% | 0.5mL |

Mix TBS, serum, blocking reagent, BSA & dH2O as shown above.

Filter through #2 Whatman filters.

Add Tween-20.

Aliquot 6mL volumes into conical tubes & store at –20°C.

Antibody dilution (AD) buffer

| stock solution | final conc | for 500mL |
| --- | --- | --- |
| 10X TBS | 1X | 50mL |
| 100% sheep serum | 5% | 25mL |
| 10% blocking reagent | 1% | 50mL |
| BSA | 1% | 0.5g |
| dH2O to vol. | -- | to 500mL |
| 100% Tween-20 | 0.1% | 0.5mL |

Mix TBS, serum, blocking reagent, BSA & dH2O as shown above.

Filter through #2 Whatman filters.

Add Tween-20.

Aliquot 6mL volumes into conical tubes & store at –20°C.

TBSTw:  1X TBS containing 0.1% Tween-20 and 0.2mM sodium azide. Pass through 0.2µm filter to remove insoluble material/contaminants.

Antibody absorption (AA) buffer

| stock solution | final conc | for 20mL |
| --- | --- | --- |
| 1X TBSTw | -- | 17mL |
| 100% sheep serum | 5% | 1mL |
| 10% blocking reagent | 1% | 2mL |
| BSA | 1% | 0.2g |
| embryo powder | -- | 0.12g |

Shake at 4°C for 30 min to rehydrate embryo powder.

Aliquot 600µL volumes & store at –20°C.

Sheep serum (must be heat-inactivated before use).

To heat inactivate:

- thaw new bottle of serum

- incubate at 70°C for 30 min

- aliquot & store at –20°C

10% Blocking reagent

| stock solution | final conc | for 100mL |
| --- | --- | --- |
| maleic acid | 100mM | 1.2g |
| 5M NaCl | 150mM | 3mL |
| dH2O to vol. | -- | to 100mL |
| Blocking reagent | 10% | 10g |

Mix maleic acid, NaCl & dH2O according to above, pH to 7.5 (note: strong buffer so difficult to pH, try using solid NaOH pellets to raise pH initially).

Add blocking reagent, microwave briefly to aid solubility (avoid boiling over, solution will be cloudy & viscous so watch carefully to ensure blocking reagent is completely dissolved in solution).

Aliquot 10 mL volumes into conical tubes & store at –20°C.

Day 3

2M Levamisole:  Dissolve 4.82g levamisole in ~7mL double-distilled H2O (total volume should equal 10mL), aliquot 200µL volumes & store stocks at –20°C.

TBSTw + levamisole:  1X TBSTw containing 2mM levamisole.

NTMT + levamisole (inhibits endogenous alkaline phosphatases):

| stock solution | final conc | for 500mL |
| --- | --- | --- |
| 1M Tris-HCl, pH 9.5 | 100mM | 50 mL |
| 5M NaCl | 100mM | 10mL |
| 1M MgCl2 | 50mM | 25mL |
| dH2O | -- | 415mL |
| 100% Tween-20 | 0.1% | 0.5 mL |
| 2M Levamisole | 2mM | 0.5mL |

Mix Tris, NaCl, MgCl2 & dH2O as shown above.

Pass through 0.2µm filter to remove insoluble material/contaminants. Store at 25°C.

DO NOT add Tween or levamisole to stock solution.

Prior to using NTMT for ISH, add 1µL of  100% Tween-20 and 1µL of 2M levamisole per 1mL NTMT stock.

3% H2O2:  1mL 30% H2O2 per 9mL PBSTw.

Abbreviations

- BSA = bovine serum albumin

- DIG = digoxigenin

- H2O2 = hydrogen peroxide

- PFA = paraformaldehyde

- PBS = phosphate-buffered saline

- PBSTw = 1X PBS + 0.1% Tween-20

- SDS = sodium dodecyl sulfate (aka lauryl sulfate)

- SSC = saline sodium citrate

- TBS = Tris-buffered saline

- TBSTw = 1X TBS + 0.1% Tween-20

Reagents and Supplies

- Anti-DIG antibody, Fab fragments, cat # 11214667001, Roche

- Blocking reagent, cat # 11096176001, Roche

- BM Purple AP substrate, precipitating, cat # 11442074001, Roche

- BSA, cat # BP1600-100, Fisher

- Formamide, cat # F5786-1L, Sigma

- Glutaraldehyde, 25% solution in H2O, cat # G6257-100ML, Sigma

- Heparin, sodium salt, cat # H3393, Sigma

- Hydrogen peroxide, 30% solution in H2O, cat # BP2633-500, Fisher

- Levamisole, cat # L9756, Sigma

- Magnesium chloride, cat # M33-500, Fisher

- Maleic acid, cat # M0375-500G, Sigma

- Paraformaldehyde, cat # 101176-014, VWR

- PBS, w/out Ca & Mg, MP Biomedicals powdered media, cat # ICN1760420, Fisher

- Proteinase K solution, 20mg/mL, biotechnology grade, cat # E195-5ML, Amresco

- RNase, cat # R6513, Sigma

- SDS, cat # S529-500, Fisher

- Sheep serum, cat # S2263-500mL, Sigma

- Sodium azide, granular, cat # S227I-100, Fisher

- Sodium chloride, cat # BP358-212, Fisher

- SSC, 20X solution, cat # S24022-4000.0, Research Products International

- Tris-HCl, cat # BP153-1, Fisher

- Tween-20, cat # BP337-100, Fisher

- Yeast tRNA, cat # 109495, Roche

- Polyester mesh, 33 micron, 12” x 24”, cat # CMY-0033-D, Small Parts Inc.