# Tissue isolation and embedding in OCT (Optimal Cutting Temperature Compound)

- Dissect tissue (e.g. bladder, ureter, etc.) rapidly and place each specimen in OCT using a plastic mold (Fisher Scientific).

- Be sure the specimen is covered in central position, not too near the top and in the desired orientation for sectioning.

- Immediately freeze the sample by placing it on dry ice.

- When the OCT is completely frozen keep the mold in dry ice, and then store long term in liquid nitrogen freezer or at  –80°C.